DJI New Pilot Experience

Key Takeaways

Phantom 4 Pro/Inspire 2 Key features

- Provides everything RTF out of the box, including 16GB SD card.

- Easy to operate with precision flight and stable hovering using dual IMU’s

- Built in Return to home, Intelligent Flight Modes, & No Fly Zones safety features

- High performance 4K/5.2K camera

- 3-axis camera stabilization

- Camera tilt control from -30 to 90 degrees

Phantom 4 Pro/Inspire 2  FAQs

| Flight Time？ | P4 Pro Up to 30min/I2 up to 27min |
| --- | --- |
| Flight Range？ | Up to 4.3 Miles (FCC Compliant) |
| Service Ceiling | Aprox. 19,000 ft. |
| How fast it flies？ | P4 Pro= 45mph; I2 = 58mph |
| Takeoff weight？ | P4 Pro = 3lbs; I2 = 8.8lbs |
| Camera quality? | P4 Pro = 16MP/ UHD4K@60p/FHD@120p; 100mbps I2 w/X4S = 16MP C4K@60p/FHD@120p; 100mbps I2 w/X5S = 20.8MP/C-DNG Raw or ProRes 5.2K@30p; 4.2gbps |

How to fly Phantom 4 Pro/Inspire 2？

- Get ready & Pre-Flight Checklist

  - Propellers and Gimbal free of obstruction check

  - Turn  on the remote controller

  - Open DJI GO 4 App

  - Turn on UAV

  - Battery Status Check (50%+)

  - Micro SD/SSD card space

- Calibration

  - Calibrate the Compass (if changing time of day, or departure location)

  - LED flight indicator to flashing Green, home point recorded

  - Verify 7+ satellite connections for proper GPS accuracy.

  - Ready to take off

- Motor Startup

  - Push both sticks down and inwards; release the sticks once the motors spin up. Or initiate Auto-Takeoff

  - Push the throttle stick upward slowly beyond the neutral position to take off

  - Monitor UAV, stay in appropriate height and direction, always maintaining visual contact.

  - Create cinematic graceful shots simply by clicking the record/shutter buttons on Radio TX

- Mission Completed

  - Land the aircraft softly. Or initiate Auto-Land

  - Turn off the aircraft by holding just the throttle stick down 3 seconds if Auto-Land was not initiated

  - Power off UAV via battery power switch

  - Turn off the remote controller

  - Synchronize album with smart device and share to them to social media.

Pre-Flight To Do’s:

- Check www.KnowBeforeYouFly.org

- Pre-flight weather check www.uavforecast.com

- Flying area check and recommendation:

  - Open and unobstructed

  - Areas with no magnetic interferences

- Compass calibration and home point record.

- Stay focused while flying, with no distractions.

- Be aware of the flight parameters, TFR’s, no fly zones, and other restricted airspace’s

- Periodically check IMU sensors are within airworthiness parameters

Phantom 4/ Inspire 1 Maintenance Notes:

- Keep away from magnetic objects and water

- Store Phantom in a cool and dry place

- Store LiPo batteries under 40% for long term (10+ days)

- Keep propellers in good conditions

*For additional instructions visit, www.youtube.com/user/TVSpecialists